# CAN DATA LOGGER

## User Manual

Version 1.0

# 1. Introduction

The CAN Data Logger is a standalone device for monitoring, recording and replaying CAN bus traffic.

### Features

- CAN bus logging to microSD card

- Standard (11-bit) and Extended (29-bit) CAN identifiers

- Configurable CAN baud rates

- Real Time Clock (RTC) timestamping

- CAN message filtering

- CAN message injection/playback

- OLED display

- Rotary encoder with integrated push button

- Firmware updates from SD card

# 2. User Controls

The logger is operated using a rotary encoder with an integrated push button.

### Rotate Encoder

- Scroll through menus

- Select menu items

- Change parameter values

### Press Encoder Knob

- Enter menus

- Select menu items

- Enter edit mode

- Save settings

# 3. Status Screen

The Status Screen is displayed after power-up.

### Information Displayed

- Software version

- CAN baud rate

- Filter status

- SD card status

- CAN injection status

- Logging status

- Received message count

Press the encoder knob to enter the Main Menu.

# 4. Main Menu

### Menu Items

- Home Screen

- Start Logging

- View Filters

- CAN Inject

- Set Date/Time

- CAN Baudrate

Rotate the encoder to highlight a menu item and press the encoder knob to select it.

# 5. CAN Logging

Select Start Logging from the Main Menu.

The logger creates a new log file on the SD card and records CAN traffic matching the configured filters.

During logging the status screen displays:

Status: LOGGING

To stop logging, select Start Logging again.

# 6. CAN Filters

Filters are loaded from the file:

FILTER.FLT

### Format

TYPE,CAN_ID

Where:

- TYPE = STD or EXT

- CAN_ID = hexadecimal CAN identifier

### Example

# Engine ECU messages
EXT,0x0CF00400

# Transmission ECU messages
EXT,0x18F00500

# Dashboard messages
STD,0x695

# Diagnostics
STD,0x7E8

### Notes

- Lines beginning with ‘#’ are treated as comments.

- Blank lines are ignored.

- Only matching messages are logged.

If FILTER.FLT is not present, all CAN messages are logged.

# 7. CAN Message Injection

Messages are loaded from:

MESSAGES.CSV

### Format

TYPE,CAN_ID,DLC,DATA…,REPEAT_TIME_MS

### Example

# Engine speed simulator
EXT,0x0CF00400,4,0x00,0x80,0x12,0x34,100

# Dashboard status
STD,0x695,4,0x00,0x01,0x02,0x03,250

# Heartbeat
STD,0x123,1,0x55,1000

### Notes

- Lines beginning with ‘#’ are treated as comments.

- Blank lines are ignored.

- The final value specifies the repeat period.

- Messages are transmitted continuously while injection is enabled.

CAN Injection cannot be enabled while logging is active.

# 8. Setting Date and Time

### Procedure

- Rotate the encoder to select a field.

- Press the encoder knob to edit the field.

- Rotate the encoder to change the value.

- Press the encoder knob to accept the value.

- Select SAVE.

- Press the encoder knob to store the new date and time.

The RTC is battery-backed and retains time when power is removed.

# 9. CAN Baud Rate

### Available Baud Rates

- 50 kbit/s

- 100 kbit/s

- 125 kbit/s

- 250 kbit/s

- 500 kbit/s

- 800 kbit/s

- 1 Mbit/s

The selected baud rate is automatically stored and restored at power-up.

# 10. Firmware Updates

### Update Procedure

- Copy FIRMWARE.BIN to the root directory of a FAT32-formatted microSD/microSDHC card.

- Insert the SD card.

- Power-cycle the logger.

The bootloader will:

- Detect the update file

- Program the application

- Verify the firmware

- Start the new application

After a successful update the file is renamed to:

FIRMWARE.OK

# 11. Bootloader Mode

During power-up the bootloader displays:

Bootloader
Hold button

Press and hold the encoder push button during startup to force the logger to remain in bootloader mode.

# 12. Troubleshooting

### SD:ERROR

The SD card could not be detected or mounted.

### No Msg File

MESSAGES.CSV could not be found.

### No Filters

FILTER.FLT could not be found.

All CAN traffic will be logged.

### Verify Failed

Firmware update verification failed.

Copy FIRMWARE.BIN to the SD card again and repeat the update process.

### Stop Logging

Logging must be stopped before changing certain settings.

# 13. Specifications

### CAN Interface

- CAN 2.0A (11-bit)

- CAN 2.0B (29-bit)

### Data Logging

- microSD / microSDHC card storage (4 GB to 32 GB recommended)

- FAT32 file system (exFAT not supported)

### Display

- 128 × 64 OLED display

### User Interface

- Rotary encoder with integrated push button

### Timekeeping

- Battery-backed RTC

### Firmware

- SD card firmware update capability

# 14. SD Card Compatibility

• Supports microSD and microSDHC cards.
• Recommended capacity: 4 GB to 32 GB.
• Cards must be formatted as FAT32.
• exFAT-formatted cards are not supported.
• Many 64 GB cards are supplied formatted as exFAT and must be reformatted to FAT32 before use.
• A Class 10 card is recommended for best logging performance.

Recommendation: For maximum compatibility, use a 4 GB to 32 GB Class 10 microSDHC card formatted as FAT32.