# J1939 Monitor & Logger

## User Manual

### Version 1.0

# 1. Introduction

The J1939 Monitor & Logger is a portable diagnostic tool for SAE J1939 networks.

The unit can:

- Monitor live J1939 parameters

- Decode PGNs and SPNs

- Log CAN traffic to an SD card

- Inject CAN messages from a CSV file

- Load multiple J1939 databases

- Update firmware from an SD card

# 2. Controls

The unit is operated using a rotary encoder with an integrated push button.

| Action | Function |
| --- | --- |
| Rotate clockwise | Move down / increase value |
| Rotate anti-clockwise | Move up / decrease value |
| Press button | Select item |
| Hold during power-up | Enter bootloader mode |

# 3. Status Screen

The status screen displays:

- Firmware version

- CAN baud rate

- Active filter count

- SD card status

- CAN injection status

- Logging status

- Loaded database

- Number of loaded SPNs

- CAN network health

Example:

J1939 LOGGER

SPN:32

CAN:250k FLT:ALL

SD:READY INJ:OFF

STOPPED DB:TRUCK

# 4. Main Menu

Home Screen

Start Logging

View Filters

CAN Inject

J1939 Monitor

J1939 Database

Settings

# 5. Logging CAN Messages

Select:

Start Logging

to begin recording CAN traffic to the SD card.

The logger automatically creates a new log file.

To stop logging:

- Return to the Main Menu.

- Select Start Logging again.

The status screen displays:

LOGGING

while recording is active.

# 6. CAN Filters

Select:

View Filters

to view the number of active CAN filters loaded from the SD card.

If no filter file is present:

FLT:ALL

is displayed and all CAN messages are logged.

Filters are loaded from the file:

FILTER.FLT

### Format

TYPE,CAN_ID

Where:

- TYPE = STD or EXT

- CAN_ID = hexadecimal CAN identifier

### Example

# Engine ECU messages
EXT,0x0CF00400

# Transmission ECU messages
EXT,0x18F00500

# 7. CAN Message Injection

The logger can periodically transmit CAN messages from:

MESSAGES.CSV

Example:

# TYPE,CAN_ID,DLC,D0,D1,D2,D3,D4,D5,D6,D7,PERIOD

EXT,0x18F00417,7,FF,FF,FF,E0,2E,FF,FF,1000

EXT,0x18FEEE17,1,7A,300

EXT,0x18FEEF17,6,FF,FF,FF,4B,FF,FF,150

EXT,0x18FEF717,5,2C,02,FF,FF,FF,,600

To enable transmission:

- Select CAN Inject.

- The status screen displays:

INJ:ON

To disable transmission:

- Select CAN Inject again.

Note:

Logging and Injection cannot operate simultaneously.

# 8. J1939 Monitor

Select:

J1939 Monitor

to display live decoded parameters.

Example:

EngineSpeed 1500 rpm

CoolantTemp 82 C

OilPressure 300 kPa

BatteryVolt 27.8 V

Values are updated automatically as messages are received.

# 9. J1939 Databases

Databases are stored in:

/J1939

Example:

GENERIC.CSV

TRUCK.CSV

ENGINE.CSV

CUMMINS_X15.CSV

The selected database determines which PGNs and SPNs are decoded.

# 10. Selecting a Database

- Select J1939 Database.

- Rotate the encoder to select a file.

- Press the encoder button.

- The database loads immediately.

The active database is shown on the status screen.

# 11. Database File Format

Example:

#PGN,SPN,DisplayName,StartBit,Length,Scale,Offset,Unit

61444,190,EngineSpeed,24,16,0.125,0,rpm

65262,110,CoolantTemp,0,8,1,-40,C

65263,100,OilPressure,24,8,4,0,kPa

Lines beginning with # are comments and ignored.

# 12. Settings

The Settings menu contains:

Date/Time

CAN Baudrate

Typical J1939 networks use:

250 kbit/s

Some systems may use:

500 kbit/s

# 13. CAN Status Messages

BUS HEAVY

Indicates increased CAN communication errors.

Possible causes:

- Missing termination resistor

- Incorrect baud rate

- Wiring fault

PASSIVE

The CAN controller has entered the error-passive state.

BUS OFF

The CAN controller has disconnected from the network due to excessive communication errors.

# 14. Firmware Updates

Update Procedure

- 1. Copy FIRMWARE.BIN to the root directory of a FAT32-formatted microSD/microSDHC card.
2. Press and hold the encoder push button.
3. While holding the button, power-cycle the unit.
4. Keep the button pressed until the bootloader starts.
The bootloader will automatically detect and install the firmware update.

- Insert the SD card.

- Power-cycle the unit.

The bootloader will:

- Detect the update file

- Program the application

- Verify the firmware

- Start the new application

After a successful update the file is renamed to:

FIRMWARE.OK

# 15. Bootloader Mode

During power-up the bootloader displays:

Bootloader

Hold button

Press and hold the encoder push button while applying power to enter bootloader mode or perform a firmware update.

# 16. Troubleshooting

No parameters displayed:

- Check CAN wiring

- Verify CAN baud rate

- Confirm the correct database is loaded

- Ensure the network is active

BUS HEAVY displayed:

- Check termination resistor

- Verify CAN baud rate

- Check CAN_H and CAN_L wiring

Database will not load:

- Verify file is in /J1939

- Verify CSV format

Firmware update fails:

- Verify filename is FIRMWARE.BIN

- Verify file is in SD card root directory

- Retry power cycle

# 18. Date and Time

Select Settings → Date/Time.
Rotate the encoder to select a field.
Press the encoder to edit the selected field.
Rotate to change the value and press again to accept.
Select SAVE to store the new date and time.
The battery-backed RTC retains the time when power is removed.

# 19. CAN Baud Rates

The following CAN baud rates are supported:
• 100 kbit/s
• 125 kbit/s
• 250 kbit/s
• 500 kbit/s
• 800 kbit/s
• 1 Mbit/s

The selected baud rate is stored in non-volatile memory and restored automatically at power-up.

# 20. Log File Format

CSV Header:
Date,Time,CAN_ID,DLC,PGN,SPN,Name,Value,Unit,Raw Data

Example:
13/07/2026,20:50:45,18FEEF17,8,65263,100,OIL,300.00,kPa,00 00 00 4B 00 00 00 00
13/07/2026,20:50:45,18F00417,8,61444,190,RPM,1500.00,rpm,00 00 00 E0 2E 00 00 00
13/07/2026,20:50:45,18FEF617,8,65270,105,INTAKE,25.00,C,41 00 00 00 00 00 00 00
13/07/2026,20:50:45,18FEEE17,8,65262,110,COOLANT,82.00,C,7A 00 00 00 00 00 00 00
13/07/2026,20:50:45,18FEF717,8,65271,168,BATT,27.80,V,2C 02 00 00 00 00 00 00

# 17. SD Card Compatibility

Supported media:
• microSD and microSDHC cards
• Recommended capacity: 4 GB to 32 GB
• Cards must be formatted as FAT32
• exFAT-formatted cards are not supported
• Many 64 GB cards are supplied formatted as exFAT and must be reformatted to FAT32 before use
• A Class 10 microSDHC card is recommended for best logging performance

Recommendation:
For maximum compatibility, use a 4 GB to 32 GB Class 10 microSDHC card formatted as FAT32.